WINTER – SEMESTER

Course Code: MCSE505P

Course-Title: – Computer Network Lab Component

DIGITAL ASSIGNMENT – IV

(LAB) Slot- L35+L36

Faculty: - SRIMATHI C (SCOPE)                                                                           Name: Nidhi Singh

Reg. No: 22MAI0015

# Implementation of RIP Routing in Cisco for Connecting Two Routers

Routing Information Protocol (RIP) is an active routing protocol that operates hop count as a routing metric to find the most suitable route between the source and the destination network. It is a distance-vector routing protocol that has an AD value of 120 and works on the Network layer of the OSI model.

### Steps to Configure and Verify Two Router Connections in Cisco Packet Tracer:

Step 1:

First, open the Cisco packet tracer desktop and select the devices given below:

IP Addressing Table :-

Use an Automatic connecting cable to connect the devices with others:-

Step 2:

Configure the PCs (hosts) with IPv4 address and Subnet Mask according to the IP addressing table given above.

- To assign an IP address in PC0, click on PC0.

- Then, go to desktop and then IP configuration and there you will IPv4 configuration.

- Fill IPv4 address and subnet mask.

Step 3: Configure router with IP address and subnet mask.

IP Addressing Table Router:-

- To assign an IP address in router0, click on router0.

- Then, go to config and then Interfaces.

- Make sure to turn on the ports

- Then, configure the IP address in FastEthernet and serial ports according to IP addressing Table.

- Fill IPv4 address and subnet mask.

Step 4: After configuring all of the devices we need to assign the routes to the routers.

To assign RIP routes to the particular router:

- First, click on router0 then Go to CLI.

- Then type the commands and IP information given below.

CLI command: network <network id>

RIP Routes for Router0 are given below:

Router(config)#router rip

Router(config-router)#network 192.168.10.0

Router(config-router)#network 10.0.0.0

Router(config-router)#

Router(config-router)#end

Router#configure terminal

RIP Routes for Router1 are given below:

Router(config-if)#router rip

Router(config-router)#network 192.168.20.0

Router(config-router)#network 10.0.0.0

Router(config-router)#

Router(config-router)#

Router(config-router)#end

Router#configure terminal

Step 5: Verifying the network by pinging the IP address of any PC.

we’ll use the ping command to do so.

- First, click on PC0 then Go to the command prompt

- then type ping <IP address of targeted node>

- as we can see in the below image we are getting replies which means the connection is working very fine

Example : ping 192.168.20.2

A simulation of the experiment is given below we are sending PDU from  PC0 to PC2 and PC1 to PC3:

Simple stop and wait ARQ implementation in C language

#include<stdio.h> int sender(); int recv(); int timer=0,wait_for_ack=-1,frameQ=0,cansend=1,t=0; int main() { int i,j; int frame[5]; printf("enter the time when data frame will be ready\n"); for(j=0;j<3;j++) { sender( i,frame); recv(i); } } int sender(int i,int frame[]) { wait_for_ack++; if(wait_for_ack==3) { if(i==frame[t]) { frameQ++; t++; } if(frameQ==0) printf("NO FRAME TO SEND at time=%d \n",i); if(frameQ>0 && cansend==1) { printf("FRAME SEND AT TIME=%d\n",i); cansend=-1; frameQ--; timer++; printf("timer in sender=%d\n",timer); } if(frameQ>0 && cansend==-1) printf("FRAME IN Q FOR TRANSMISSION AT TIME=%d\n",i); if(frameQ>0) t++; } printf("frameQ=%d\n",frameQ); printf("i=%d t=%d\n",i,t); printf("value in frame=%d\n",frame[t]); return 0; } int recv(int i ) { printf("timer in recvr=%d\n",timer); if(timer>0) { timer++; } if(timer==3) { printf("FRAME ARRIVED AT TIME= %d\n",i); wait_for_ack=0; timer=0; } else printf("WAITING FOR FRAME AT TIME %d\n",i); return 0; }

Output:-